FAULT TOLERANT SYSTEMS

LABORATORY PROJECT REPORT

MASTER OF TECHNOLOGY

Digital Systems and Computer Electronics

“LFSR based Test Generation”

Submitted by

P. ROHAN

25011D0613

In Presence of

Dr. T. Satya Savithri, Professor

Department of Electronics and Communication Engineering

Jawaharlal Nehru Technological University Hyderabad

University College of Engineering, Science & Technology

Hyderabad

LFSR based Test Generation

Aim:

to design and implement LFSR based test generation on Boolean Board and observe its functionality.

Appratus / requirements:

1. AMD Vivado

2. Boolean Board

Theory:

Linear Feedback Shift Register:

A Linear Feedback Shift Register (LFSR) is a sequential digital circuit used to generate a sequence of binary values that appears random. It consists of a series of flip-flops connected as a shift register, where the input to the first flip-flop is formed by applying the XOR operation to selected outputs (called taps) of the register. Although the output looks random, it is completely deterministic and repeats after a fixed number of clock cycles.

The operation of an LFSR is simple. On each clock pulse, all bits shift by one position, and the feedback bit generated from the selected taps is inserted into the first flip-flop. The sequence length depends on the feedback polynomial. When a primitive polynomial is used, an n-bit LFSR generates a maximum-length sequence of 2ⁿ − 1 unique states before repeating, excluding the all-zero state.

LFSRs are widely used in digital systems because they require very little hardware and operate at high speed. Common applications include pseudo-random number generation, Cyclic Redundancy Check (CRC) computation, Built-In Self-Test (BIST), data scrambling and descrambling, stream ciphers, spread-spectrum communication, and fault-tolerant testing. Their simplicity, efficiency, and low power consumption make them popular in FPGA and ASIC designs.

The Boolean board:

The Boolean FPGA Board by Real Digital is an educational trainer board based on an AMD-Xilinx Spartan-7 FPGA, typically from the XC7S series in a CS324 package. It is designed specifically for teaching and learning digital logic design, HDL programming (Verilog/VHDL), and introductory FPGA-based system development. The board provides a low-power, cost-effective FPGA platform that allows students to move from basic Boolean logic experiments to more advanced embedded and communication-based designs.

The board integrates a rich set of on-board input and output peripherals, making it suitable for laboratory use without requiring additional external hardware. User inputs include slide switches, push buttons, and a potentiometer, which are commonly used for implementing and testing Boolean functions, counters, and finite state machines. For outputs, the board features multiple LEDs and an 8-digit seven-segment display, enabling clear visualization of logic states, numerical data, and timing behavior during experiments.

In terms of connectivity, the Boolean board supports several communication and expansion interfaces. It includes USB-UART for FPGA programming and serial communication with a PC, PMOD connectors for interfacing external modules, and additional interfaces such as HDMI output, audio output, servo headers, and a BLE (Bluetooth Low Energy) module. These features allow students to explore real-world applications such as UART communication, display control, motor control, and basic multimedia output.

The board is powered through USB and includes an on-board clock oscillator (typically 100 MHz) along with the required voltage regulation circuitry, ensuring stable and reliable operation. The Boolean FPGA Board is supported by the Xilinx Vivado Design Suite, which is widely used in industry and academia, helping students gain practical experience with professional FPGA development tools.

Program Code:

module lfsr #(

parameter N=8

)(

input clk,

input rst_n,

output reg [N-1:0] q

);

reg [26:0] count=0;

reg clk1hz=0;

always @(posedge clk) begin

if(count==49_999_999) begin

count<=0;

clk1hz<=~clk1hz;

end

else

count<=count+1;

end

wire feedback;

assign feedback=q[N-1]^q[5]^q[4]^q[3];

always @(posedge clk1hz or negedge rst_n) begin

if(!rst_n)

q<=8'b00000001;

else

q<={feedback,q[N-1:1]};

end

endmodule

Testbench:

`timescale 1ns / 1ps

module lfsr_tb;
	parameter N = 8;
	parameter CLK_PERIOD = 20; // 50 MHz clock -> 20ns period
	reg clk;
	reg rst_n;
	wire [N-1:0] q;
	lfsr #(
    		.N(N)
	) uut (
    		.clk(clk),
    		.rst_n(rst_n),
   		 .q(q)
	);
	always begin
   		clk = 1'b0;
   		#(CLK_PERIOD/2);
    		clk = 1'b1;
    		#(CLK_PERIOD/2);
	end
	initial begin
    		rst_n = 1'b0;
    		#(CLK_PERIOD * 3);
    		rst_n = 1'b1;
    		$display("Reset released. LFSR initialized to: %b", uut.q);
    		#(CLK_PERIOD * 40);
    		$display("Applying reset mid-simulation...");
    		rst_n = 1'b0;
   		#(CLK_PERIOD * 2);
    		rst_n = 1'b1;
    		#(CLK_PERIOD * 20);
    		$display("Simulation complete.");
    		$finish;
	end
	initial begin
    		$monitor("Time = %0t | Reset = %b | LFSR Output (q) = %b (%d)", 
             	$time, rst_n, q, q);
	end

endmodule

### Constraints(.XDC):

set_property -dict{PACKAGE_PIN F14 IOSTANDARD LVCMOS33} [get_ports {clk}];

set_property -dict{PACKAGE_PIN A4 IOSTANDARD LVCMOS33} [get_ports {q[7]}];

set_property -dict{PACKAGE_PIN B4 IOSTANDARD LVCMOS33} [get_ports {q[6]}];

set_property -dict{PACKAGE_PIN A3 IOSTANDARD LVCMOS33} [get_ports {q[5]}];

set_property -dict{PACKAGE_PIN B3 IOSTANDARD LVCMOS33} [get_ports {q[4]}];

set_property -dict{PACKAGE_PIN A2 IOSTANDARD LVCMOS33} [get_ports {q[3]}];

set_property -dict{PACKAGE_PIN B2 IOSTANDARD LVCMOS33} [get_ports {q[2]}];

set_property -dict{PACKAGE_PIN C3 IOSTANDARD LVCMOS33} [get_ports {q[1]}];

set_property -dict{PACKAGE_PIN E6 IOSTANDARD LVCMOS33} [get_ports {q[0]}];

Result:

The configurable LFSR was implemented on Boolean board and observed its functionality.

### Applications:

- Built-In Self-Test (BIST) – Test Pattern Generator (TPG)

- Pseudo-random number generation

- Scrambling / Descrambling

- Error detection and correction

- Cryptographic preprocessing

- Communication systems

### Conclusion:

The configurable Linear Feedback Shift Register (LFSR) was successfully designed and implemented on a Boolean board. The design allowed the feedback polynomial and seed value to be externally programmed, enabling the generation of different pseudo-random sequences using the same hardware. The correct operation of shifting, feedback logic, and sequence generation was verified for different configurations. This implementation demonstrated the flexibility of configurable LFSRs and their importance in applications such as Built-In Self-Test (BIST), test pattern generation, and digital system testing.